# OSA

Pros

- It looks a lot more like a normal RTOS than CCS RTOS

- It's a little crude, but think it's a better option, if we go RTOS

- It's free

- Should work on the XC8

Cons

- It is co-operative, not pre-emptive.

- It has not been updated in 8.5 years

- There is no supported port to XC8.  Latest supported compiler is C18 (MPLAB compiler)

I think I could get it working with XC8 in 4 hours or so. Looks pretty straight forward.  I suggest that we consider a "after so much time is spent, abandon OSA and go bare metal" mentality.

I've attached an example program (minus the project specific files) so you can quickly see how we would use OSA.

# CCS IDE+buildchain+RTOS

God bless it. So, we need the IDE. Without it, we don't get the RTOS...this thing is officially a pain.

Trying to use the $200 version will definitely cost ASL/Invacare more money now as I'd have to rip up a lot of code. Luckily, I've only spent 30 minutes so far trying to get the code to work with the $200 version.  I can shelve my changes and revert.

If we stick with the $200 solution, my suggestion is we go with this RTOS the OSA RTOS:

http://www.ccsinfo.com/forum/viewtopic.php?t=39433
http://wiki.pic24.ru/doku.php/en/osa/ref/download/intro

I'm going to see if I can be "upgraded" from the $200 build chain to the $500 without taking a $200 hit.

I dug into the CCS RTOS this morning for a few minutes. Here's the overview:

- It's very very basic. Nothing wrong with that, I think the RTOS constructs it provides is sufficient.

- We get the following functionality:

  - task yield until expression is true

  - Counting sems

  - Round robin RTOS (think I said it was a pre-emptive before, but it's not).

  - No ties to ISRs.  So, ISRs must be monitored and acted upon outside of the RTOS.

I noticed that there is an EEPROM driver. It won't take long, but I will be creating a BSP file for CCS so as to maintain the already written code for XC8.

I really don't like the CCS IDE much.  The fact that I struggled to get PICkit3 working with it bothered me to the point that I spent a few minutes trying to plug the CCS build chain into MPLAB X.
Verdict: not a complete success, but I feel better about plugging CCS into MPLAB X than I do about using CCS IDE going forward.  I was able to quickly get the plug in installed and building code
inside MPLAB X using the CCS build chain.  I got stuck on MPLAB X being able to detect the PIC16F887 processor on my eval kit. I suspect the kit may just be bad, but I am not sure. I have not used this kit in years.

However, I do largely prefer MPLAB X over CCS IDE. My suggestion is that we move forward with MPLAB X as the IDE and simply use CCS IDE to auto-generate code when it makes sense to do so.  Please advise on this one.

[TJP] ADDED

I have a license for this still…so the cost isn’t a concern. However, I have the cheapo version. This means we cannot build more than 1 source file per project.  Very very annoying.

# FreeRTOS

I also dug into FreeRTOS on the PIC18F4550. Right on FreeRTOS's web site it says using an RTOS on this part is not recommended due to the fact that memory is segmented. See https://www.freertos.org/a00090.html#MICROSEMI, search for PIC18 inside that page.

Also, MCC (code generator tool) does not support this part.

I think running FreeRTOS is still worthwhile and there are decent instructions and base ports to get started.  Please advise on whether you want me to go down the FreeRTOS route or just go bare metal.

# 32-bit PIC MCU

I am working on a project where an ancient PIC18F4550 is being used for
no other reason than the hardware guy who picked it is comfortable with
it.  Unfortunately, MCC and FreeRTOS do not support it really at all. 
MCC I could kind of deal with by using legacy library support. But I
want an RTOS in this thing. So, it seems like a PIC32MX130F064D will
likely get the job done for ~60% of the cost in small quantity.